Data Accuracy & Upload-Interval Issue: InHand Cloud Reports Stale Cellular State

To: InHand Networks Support / Engineering From: Deecell (account: andy@deecell.com) Date: 2026-06-25 Re: (1) Device Manager upload intervals are capped at 1 hour; (2) even at that cap, location/cell data is stale and does not match the device's live radio state

# Scope of this request (please read first)

This is a data-accuracy and configuration issue, not a request for a GPS/tracking product. We are not asking the router to be a vehicle tracker. We are reporting two concrete problems with the cellular network data your cloud already collects — serving cell ID (cid), location area code (lac), carrier (mcc/mnc), signalStrength, and cell-tower location (LBS):

- Your Device Manager service will not upload this data more often than once per hour (the config field's valid range is 1–24 hours), and

- Even at that 1-hour setting, the data we receive is far older than an hour and does not reflect the device's actual current cell.

The router inherently knows its serving cell and signal at all times (it must, to keep its data connection up), so this is a defect/limitation in the cloud upload pipeline and its configurable cadence, not a missing hardware feature.

# Finding 1 — The upload interval is capped at 1 hour (this is the core blocker)

On DCL-Moeck-Fleet (IR300), under Service Manager → Device Manager (Server: iot.inhandnetworks.com), the relevant settings are:

| Setting | Current value | Meaning |
| --- | --- | --- |
| LBS info Upload Interval | 1 Hour | Cell-tower (LBS) location upload cadence |
| Series Info Upload Interval | 1 Hour | Signal / cell telemetry time-series upload cadence |
| Channel Keepalive | 30 Seconds | — |

Both are already at the minimum. Attempting to set either below 1 returns the error: "valid values are 1–24" (hours). So the fastest the router will push cell/LBS/signal telemetry to your cloud — which is what our API (GET /api/devices?verbose=100, GET /api/devices/{id}/signal) reads — is once per hour.

What we need: a supported way to upload LBS and Series Info at sub-hour intervals — ideally ~1 minute (30–60 s). Specifically:

- Is there a CLI, advanced config, or firmware option that allows an upload interval below 1 hour?

- If not, can the platform be updated to accept seconds/minutes for these fields?

- If 1 hour is a hard platform limit, please tell us directly so we can plan around it.

# Finding 2 — Even at the 1-hour cap, the data is stale and wrong

This is independent of the interval cap and, we believe, a genuine bug.

## 2a. Location is days stale, not 1 hour

Baseline reading (2026-06-25 22:10 UTC) from your API for DCL-Moeck-Fleet:

- location.source = "cellTower", `location.time = 2026-06-21T01:03:08Z` — i.e., ~4.5 days old, despite the LBS upload interval being set to 1 hour.

- Across all 48 of our devices, 0 report `location.source: "gps"`, and location.time is routinely multiple days stale.

If LBS were honoring its 1-hour interval, location should be at most ~1 hour old. It is not. Why does LBS exceed its own configured upload interval?

## 2b. The serving cell does not change across a 22-mile drive

We ran a controlled drive: one router traveled 22 miles from West Spokane, WA to the Idaho state line on I-90 and back. Over that drive your API reported:

- `info.cid = 107713764` — UNCHANGED the entire 22-mile leg.

- `lac = 37122` — UNCHANGED the entire 22-mile leg (including crossing into the Idaho market).

- The signal-block signalStrength.cid changed only once in 22 miles.

A vehicle driving 22 miles on an interstate passes through dozens of cell sectors; the serving cid must change many times and lac should change at least once. A single change (and zero change in the info block) means the uploaded values are cached/stale, not the device's actual current serving cell.

We are not citing mcc/mnc (310/410, AT&T) as evidence — that is expected to stay constant if the device remained on AT&T. The unambiguous proof is `cid` (and lac), which cannot remain fixed across 22 miles.

## 2c. Telemetry freezes during weak signal, then jumps

When the device was on a weaker tower near the Idaho border, the telemetry timestamp stopped advancing for 35+ minutes (parked or moving), then jumped forward in a single step once back in stronger coverage. This suggests the cloud serves a last-known cached value rather than buffering timestamped samples and uploading them on reconnect (store-and-forward).

# What we need from InHand (summary)

Primary

- Allow sub-hour upload intervals for LBS info and Series Info (your field caps at 1–24 hours; we need ~1 minute). Is there a CLI/advanced/firmware path, or can the limit be changed?

- Fix LBS not honoring its interval — explain why location.time is days stale when LBS is set to 1 hour.

- Report the live serving cell — info.cid/lac (and signalStrength.cid) must reflect the device's current tower at each upload, not a cached value.

- Store-and-forward — buffer timestamped telemetry during weak-signal gaps and upload on reconnect, instead of serving a frozen last-known value.

Secondary (optional)

- If the IR300 (product 302FQ38-WLAN, serial RF3022532644350) has a GNSS/GPS module, how do we enable it so location.source becomes "gps" with continuous fixes? Helpful, but not required to fix items 1–4.

## Questions

- How do we set LBS/Series upload intervals below 1 hour (CLI, firmware, or platform change)? Is 1 hour a hard limit?

- Why is location.time days stale when LBS upload is set to 1 hour?

- Why do info.cid/lac stay fixed across a 22-mile drive?

- Does the platform support store-and-forward of telemetry across connectivity gaps?

- (Optional) Does this IR300 SKU have GNSS, and how is it enabled?

We can provide our full raw drive logs (30-second API samples with timestamps, cid, lac, mcc, mnc, and all signalStrength fields) and a screenshot of the Device Manager settings, and we can re-run the test on any device you specify. Our ask is simple: let the router upload its real, current cellular state at a useful cadence.

Thank you, Deecell

# Technical reference

Example device (used in the drive test):

- Name: DCL-Moeck-Fleet

- Model: IR300 · Product no.: 302FQ38-WLAN · Serial: RF3022532644350

- Firmware: swVersion V3.5.99, bootVersion 1.1.3.r4956, hwVersion V1.0

- Device Manager service config: LBS info Upload Interval = 1 Hour (min, range 1–24), Series Info Upload Interval = 1 Hour (min, range 1–24), Channel Keepalive = 30 s

- Internal poll config seen via API: { "sync": 2, "timeout": 300000, "ackTimeout": 120000, "ackRetries": 3 }

API endpoints we read: GET /api/devices?verbose=100 (bulk) and GET /api/devices/{id}/signal.

Baseline reading — 2026-06-25 22:10 UTC:

| Field | Value | Age |
| --- | --- | --- |
| location.time | 2026-06-21T01:03:08Z (cellTower) | ~4.5 days |
| signalStrength.ts | 2026-06-25T22:00:00Z | on the hour |
| info.cid / lac | 108634129 / 37122 | — |
| info.updatedAt | 2026-06-25T21:43:55Z | ~27 min |

Drive-test timeline (UTC; local Pacific = UTC−7), West Spokane → ID border and back:

| UTC | Event | info.cid | lac | signalStrength.cid | Signal | Telemetry ts |
| --- | --- | --- | --- | --- | --- | --- |
| 16:23 | Leave shop (W Spokane) | 107713764 | 37122 | 158248449 | rssi −71, sinr +12 | 16:15 |
| 16:37 | Driving I-90 E | 107713764 | 37122 | 107567895 | rssi −85, sinr −3 | 16:30 |
| 16:54 | Arrive ID border (~22 mi) | 107713764 | 37122 | 107567895 | weak | 16:30 (then froze) |
| 17:05 | Parked ID (weak signal) | 107713764 | 37122 | 107567895 | rsrp −120, sinr −4 | 16:30 (35+ min stale) |
| 17:21 | Back near Spokane | 107713764 | 37122 | 107594505 | rssi −79, sinr 0 | jumped → 17:00 |

The smoking gun: info.cid and lac are identical at 16:23 (West Spokane) and 16:54 (Idaho state line, 22 miles later). A live radio cannot stay on one cell across that distance.